BWD Report For the Bathing Season 2017

Luxembourg

The report gives a general overview of information acquired from the reported data, based on provisions of the Bathing Water Directive. The reporting process is described below, as well as state and trends of bathing water quality in Luxembourg.

# BWD reporting in the season 2017

Bathing waters of Luxembourg in 2017

Total reported 	12

Coastal	0

Inland	12

Max season period 	153 days

1 May to 30 Sep

Samples taken	72

Share of bathing waters 	100 %

with good or excellent

water quality

Reporting under	2007

Directive 2006/7/EC since

In the 2017 bathing season, 12 bathing waters have been reported in Luxembourg. For each bathing water, five groups of parameters have been delivered:

- identification data – including name, location, coastal, inland or transitional type of bathing water and availability to bathers;

- seasonal data – including season start and end, national quality classification in the recent season, potential management measures and changes that are likely to affect the classification of the bathing water;

- monitoring results – disaggregated numerical values of two microbiological parameters – intestinal enterococci and Escherichia coli (also known as E. coli), recorded at each water sample taken;

- abnormal situation periods – periods of an event or combination of events impacting on bathing water quality, during which monitoring calendar may be suspended; reporting is optional;

- short-term pollution periods – measurable events of microbiological contamination; reporting is optional.

The authorities of Luxembourg report data according to the new BWD (2006/7/EC) since the season 2007.

Altogether, 12 bathing waters have been reported – 0.1% of all bathing waters in Europe. One bathing water has been newly reported in the recent season. All bathing waters in Luxembourg are inland. 72 samples were taken at bathing waters throughout the season – 6 per bathing water on average.

The bathing season period was from 1 May to 30 September, i.e. 153 days.

Detailed information on bathing waters is available from national portal at https://eau.public.lu/actualites/2011/03/Profil_baignade/.

# Assessment methodology

During the bathing season, water samples are taken and analysed for two bacteria, Escherichia coli and intestinal enterococci which may indicate the presence of pollution, usually originating in sewage, livestock waste, bird faeces etc. The results of the analysis are used to assess the quality of the bathing waters concerned and to provide information to the public on the quality of water in the bathing sites concerned.

The monitoring requirements under the Directive are:

- taking a pre-season sample (taken shortly before the start of the bathing season) ;

- a minimum of four samples per season;

- a minimum of one sample per month.

If these rules are satisfied, the bathing water is categorised as 'sampling frequency satisfied'. If not all monitoring requirements are fulfilled the bathing water is categorised as 'not enough samples'. 100.0% of bathing waters met the described monitoring requirements set by the Directive, while the rest did not satisfy monitoring requirements for different reasons: being new; having changed environmental conditions that might affect water quality classification; closed; not monitored due to legal issues, physical inaccessibility to the site etc. Table 1 shows the statistics of bathing waters according to monitoring requirements.

Table 1: Bathing waters in 2017 according to compliance with BWD monitoring provisions

|  | Count | Share of total [%] |
| --- | --- | --- |
| BWs with sampling frequency satisfied (and are not new, are not subject to changes or were not closed in 2017) These bathing waters have been monitored according to provisions and have complete dataset from the last assessment period. They have been quality-classified (excellent, good, sufficient, poor). | 12 | 100.0% |
| BWs with sampling frequency not satisfied (and are not new, are not subject to changes or were not closed in 2017) These bathing waters exist throughout the last assessment period but have not been monitored throughout the period according to provisions for various individual reasons. They may be quality-classified if there is an adequate volume of samples available for credible classification. | 0 | 0.0% |
| BWs that are new, subject to changes or closed in 2017 These bathing waters do not have complete dataset for the last assessment period because they are new, have been subject to changes (that are likely to affect the classification of the bathing water) or have been closed. They cannot be quality-classified. | 0 | 0.0% |
| Total number of bathing waters in 2017 | 12 | 100% |

Bathing waters where sampling frequency was not satisfied can still be quality assessed if at least four samples per season (three samples if the season does not exceed eight weeks or the region is subject to special geographical constraints) are available and equally distributed throughout the season. Assessment of bathing water quality is possible when the bathing water sample dataset is available for four consecutive seasons. Bathing waters are accordingly classified to one of the bathing water quality classes (excellent, good, sufficient, or poor).

The classification is based on pre-defined percentile values for microbiological enumerations, limiting the classes given in Annex I of the Directive. The Directive defines different limit values for coastal and inland waters.

Quality assessment is not possible for all bathing waters. In these cases, they are instead classified as either:

• not enough samples;

• new;

• changes;

• closed.

# Bathing water quality

The results of the bathing water quality in Luxembourg throughout the past period are presented in Figure 2. The previous reports are available on the European Commission's bathing water quality website and the European Environment Agency's bathing water website.

## Coastal bathing waters

There are no coastal bathing waters in Luxembourg.

## Inland bathing waters

All existing inland bathing waters were of at least sufficient water quality in 2017. See Appendix 1 for numeric data.

Figure 2: Inland bathing water quality trend in Luxembourg. Note: the “At least sufficient” class also includes bathing waters of “Excellent” quality class, the sum of shares is therefore not 100%.

# Information regarding management and other issues

Since the 2011 bathing season, the bathing water profiles are published for each bathing water determining the special protection measures and monitoring programmes the state of bathing water.

Bathing water profile contains:

- A description of the physical, geographical and hydrological characteristics of the bathing water that could be a source of pollution;

- Identification and evaluation of sources of pollution that might affect bathers' health;

- Evaluation of the potential for proliferation of cyanobacteria and macroalgae;

- The location of the monitoring point.

If the bathing water profile identified a source of pollution, it should also inform about:

- The nature, frequency and duration of anticipated pollution which can be expected;

  - The details of any remaining causes of pollution, including management measures taken and the timetable for their elimination.

# Bathing water quality assessment presentation in online viewers

The European bathing water legislation focuses on sound management of bathing waters, greater public participation and improved information dissemination. More on the bathing and other water legislation can be found on the European Commission's website: http://ec.europa.eu/environment/water/index_en.htm.

The bathing water section of the Water Information System for Europe (WISE) which is accessible at the EEA bathing water website (http://www.eea.europa.eu/themes/water/interactive/bathing/state-of-bathing-waters) allows users to view the bathing water quality at more than 21 000 coastal and inland sites across Europe. The WISE bathing water quality data viewer combines text and graphical visualisation, providing a quick overview of the bathing water's locations and achieved quality. Having access to bathing water information, citizens are encouraged to make full use of it and participate with their comments.

Appendix 1: Results of bathing water quality in Luxembourg from 2014 to 2017

Table 2: Bathing waters in the season 2017 according to quality

|  |  | Total number of bathing waters | Excellent quality |  | At least sufficient quality |  | Poor quality |  | Quality classification not possible: not enough samples /new bathing waters/bathing waters subject to changes/closed |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Count | % | Count | % | Count | % | Count | % |
| Total | 2014 | 11 | 11 | 100.0 | 11 | 100.0 | 0 | 0.0 | 0 | 0.0 |
| Total | 2015 | 11 | 11 | 100.0 | 11 | 100.0 | 0 | 0.0 | 0 | 0.0 |
| Total | 2016 | 11 | 11 | 100.0 | 11 | 100.0 | 0 | 0.0 | 0 | 0.0 |
| Total | 2017 | 12 | 12 | 100.0 | 12 | 100.0 | 0 | 0.0 | 0 | 0.0 |

Note: the class “At least sufficient” also includes bathing waters which are of excellent quality, the sum of shares is therefore not 100%.

Appendix 2: Bathing water quality map

Directive BWD 2006/7/EC, available at http://eur-lex.europa.eu/LexUriServ/LexUriServ.do?uri=OJ:L:2006:064:0037:0051:EN:PDF

See the BWD Data Dictionary for detailed explanations: http://dd.eionet.europa.eu/datasets/3294#tables

The methodology used by the EC and the EEA is described here, while results of assessment by national authorities may differ in individual cases.

A pre-season sample is taken into a sum of samples per season.

Three samples are sufficient if the season does not exceed eight weeks or the region is subject to special geographical constraints.

If, for any reason, it is not possible to take the sample at the scheduled date, a delay of four extra days is allowed. Thus, the interval between two samples should not exceed 31 + 4 days.

Not enough samples have been provided throughout the last assessment period (the last four bathing seasons or, when applicable, the period specified in Article 4.2 or 4.4).

Classification not yet possible because bathing water is newly identified and a complete set of samples is not yet available.

Classification is not yet possible after changes that are likely to affect the classification of the bathing water.

Bathing water is closed temporarily or throughout the bathing season.

http://ec.europa.eu/environment/water/water-bathing/index_en.html

http://www.eea.europa.eu/themes/water/status-and-monitoring/state-of-bathing-water